4.  MATLAB SIMULATION OF SUPERHETERODYNE RECEIVER

FOR AM COMMUNICATION SYSTEM WITH FREQUENCY

MIXING AND FILTERING

PROGRAM: :
clc;

clear;

close all;

% Parameters

fs = 100000;

t = 0:1/fs:0.02;

fm = 500; % Message frequency

fc = 10000; % Carrier frequency

fLO = 14550; % Local Oscillator (IF = 4550 Hz)

% Message Signal

m = cos(2*pi*fm*t);

% Carrier Signal

c = cos(2*pi*fc*t);

% AM Signal

am = (1 + m).*cos(2*pi*fc*t);

% Local Oscillator

LO = cos(2*pi*fLO*t);

% Frequency Mixing

mixed = am .* LO;

% IF Filter

[b,a] = butter(4,5000/(fs/2));

IF = filter(b,a,mixed);

% Envelope Detection

env = abs(hilbert(IF));

% Display

figure;

subplot(4,1,1)

plot(t,m)

title('Message Signal')

grid on

figure;

subplot(5,1,2)

plot(t,c)

title('Carrier Signal')

grid on

subplot(5,1,3)

plot(t,am)

title('AM Signal')

grid on

subplot(5,1,4)

plot(t,IF)

title('Intermediate Frequency (IF) Signal')

grid on

subplot(5,1,5)

plot(t,env)

title('Recovered Signal')

grid on