6.  GENERATION AND ANALYSIS OF FREQUENCY MODULATED

(FM) AND OF PHASE MODULATED (PM) SIGNALS USING MATLAB

PROGRAM:
% Calculation and Verification of Frequency Deviation

% and Modulation Index of FM Signal

clc;

clear;

close all;

%% Message Signal Parameters

Am = 2; % Message amplitude (V)

fm = 500; % Message frequency (Hz)

%% Carrier Signal Parameters

Ac = 1; % Carrier amplitude (V)

fc = 10000; % Carrier frequency (Hz)

%% FM Parameters

kf = 250; % Frequency sensitivity (Hz/V)

fs = 100000; % Sampling frequency (Hz)

T = 0.01; % Simulation duration (s)

%% Time Vector

t = 0:1/fs:T;

%% Message Signal

m = Am*cos(2*pi*fm*t);

%% Frequency Deviation

delta_f = kf*Am;

%% Modulation Index

beta = delta_f/fm;

%% FM Signal

fm_signal = Ac*cos(2*pi*fc*t + beta*sin(2*pi*fm*t));

%% Display Parameters

fprintf('Carrier Frequency (fc) = %.0f Hz\n',fc);

fprintf('Message Frequency (fm) = %.0f Hz\n',fm);

fprintf('Message Amplitude (Am) = %.0f V\n',Am);

fprintf('Frequency Sensitivity (kf) = %.0f Hz/V\n',kf);

fprintf('\nFrequency Deviation (Delta f) = %.2f Hz\n',delta_f);

fprintf('Modulation Index (Beta) = %.2f\n',beta);

%% Verification

beta_verify = delta_f/fm;

fprintf('Verified Modulation Index = %.2f\n',beta_verify);

%% Verification Check

tolerance = 1e-6;

if abs(beta - beta_verify) < tolerance

fprintf('\nVerification Successful\n');

else

fprintf('\nVerification Failed\n');

end

%% Plotting

figure;

subplot(2,1,1);

plot(t,m,'LineWidth',1.5);

grid on;

title('Message Signal');

xlabel('Time (s)');

ylabel('Amplitude (V)');

subplot(2,1,2);

plot(t,fm_signal,'LineWidth',1);

grid on;

title('FM Signal');

xlabel('Time (s)');

ylabel('Amplitude (V)');

OUTPUT